# Inter-LAN Communication (Router Between Two LANs)

Question: Connect Admin and Accounts departments on different floors using a router.

## Topology Flowchart:

[PC-A] ─ [Switch-A] ─ (G0/0) [Router0] (G0/1) ─ [Switch-B] ─ [PC-B]

## Steps:

Assign IPs: Admin LAN - 192.168.10.0/24, Accounts LAN - 192.168.20.0/24.
Router G0/0 - 192.168.10.1, Router G0/1 - 192.168.20.1.

## Router CLI Configuration:

enable
configure terminal
interface g0/0
 ip address 192.168.10.1 255.255.255.0
 no shutdown
exit
interface g0/1
 ip address 192.168.20.1 255.255.255.0
 no shutdown
exit
end
write

## Verification:

Ping 192.168.20.2 from 192.168.10.2.

## Concept:

Router connects two LANs so they can communicate.