# Subnetting

Question: Divide 192.168.10.0/24 into two subnets.

## Topology Flowchart:

[Dept A PCs] ─ [Switch A] ─ (G0/0) [Router] (G0/1) ─ [Switch B] ─ [Dept B PCs]

## Steps:

Subnet 1: 192.168.10.0/25, Subnet 2: 192.168.10.128/25.
Router G0/0: 192.168.10.1, Router G0/1: 192.168.10.129.

## Router CLI Configuration:

enable
configure terminal
interface g0/0
 ip address 192.168.10.1 255.255.255.128
 no shutdown
exit
interface g0/1
 ip address 192.168.10.129 255.255.255.128
 no shutdown
exit
end
write

## Verification:

Ping between PCs in both subnets to confirm communication.

## Concept:

Subnetting divides a large network into smaller, manageable subnets.