# RIP (Dynamic Routing)

Question: Configure RIP on three routers for automatic routing updates.

## Topology Flowchart:

[LAN A] ─ [R1] ─ [R2] ─ [R3] ─ [LAN C]

## Steps:

RIP v2 shares routes automatically between routers.

## Router CLI Configuration:

enable
configure terminal
router rip
 version 2
 no auto-summary
 network 10.10.0.0
end
write

## Verification:

Use 'show ip route' to see RIP-learned routes.

## Concept:

RIP allows routers to automatically exchange route information.