# VOIP (Voice over IP)

Question: Configure VoIP using router as Call Manager Express (CME).

## Topology Flowchart:

[IP Phone1] ─ [PoE Switch] ─ (G0/0) [Router CME] (G0/1) ─ [PoE Switch] ─ [IP Phone2]

## Steps:

Router provides DHCP and CME services for two IP phones.

## Router CLI Configuration:

enable
configure terminal
interface g0/0
 ip address 10.10.90.1 255.255.255.0
 no shutdown
exit
ip dhcp pool VOICE
 network 10.10.90.0 255.255.255.0
 default-router 10.10.90.1
telephony-service
 ip source-address 10.10.90.1 port 2000
 max-ephones 5
 max-dn 5
 auto assign 1 to 2
exit
ephone-dn 1
 number 101
ephone-dn 2
 number 102
end
write

## Verification:

Dial 102 from 101 to confirm voice communication.

## Concept:

VoIP allows internal phone communication using IP packets and router as call manager.