# DHCP (Dynamic IP Assignment)

Question: Configure DHCP using a router to automatically assign IPs.

## Topology Flowchart:

[PC0] ─ [Switch] ─ (G0/0) [Router] ─ [PC1]

## Steps:

1. Connect PCs and router.
2. Configure DHCP pool on router.
3. PCs obtain IP automatically.

## Router CLI Configuration:

enable
configure terminal
interface g0/0
 ip address 192.168.50.1 255.255.255.0
 no shutdown
exit
ip dhcp excluded-address 192.168.50.1 192.168.50.10
ip dhcp pool OFFICE
 network 192.168.50.0 255.255.255.0
 default-router 192.168.50.1
 dns-server 8.8.8.8
end
write

## Verification:

Use ipconfig on PC to verify IP assignment.

## Concept:

DHCP allows automatic IP allocation from the router.

1) DHCP Lab — Router as DHCP Server (Q8)

Goal

Automatically assign IPs, gateway and DNS to multiple PCs on a LAN using the router as DHCP server.

Topology (ASCII flowchart)

[PC1] ──┐

[PC2] ──┤

[PC3] ──┤── [Switch0] ── (Gi0/0) Router0

[PC4] ──┘

IP plan

- LAN network (private): 10.10.20.0 /24

  - Router (gateway): 10.10.20.1 /24 (GigabitEthernet0/0)

  - DHCP pool to hand out: 10.10.20.11 – 10.10.20.200

- Excluded addresses: 10.10.20.1 – 10.10.20.10 (reserved for router, servers, printers)

Step-by-step (Packet Tracer actions)

- Place Router0 (1941/2911), Switch0 (2960), and 4 PCs.

- Connect each PC → Switch0 (Copper straight-through). Connect Switch0 → Router0 Gi0/0 (straight-through).

- Power devices on (default). No PoE needed for PCs.

Router CLI — copy/paste

enable

configure terminal

! 1) Configure LAN interface (gateway)

interface GigabitEthernet0/0

description TO_LAN

ip address 10.10.20.1 255.255.255.0

no shutdown

exit

! 2) Exclude reserved addresses from DHCP

ip dhcp excluded-address 10.10.20.1 10.10.20.10

! 3) Create DHCP pool

ip dhcp pool OFFICE

network 10.10.20.0 255.255.255.0

default-router 10.10.20.1

dns-server 8.8.8.8

lease 7

end

write

What each command does (plain language)

- interface GigabitEthernet0/0 ... ip address 10.10.20.1 255.255.255.0 sets the router’s LAN IP (the gateway).

- ip dhcp excluded-address 10.10.20.1 10.10.20.10 prevents the DHCP server from handing out .1–.10.

- ip dhcp pool OFFICE + network defines the DHCP scope (which network and mask to serve).

- default-router tells clients what gateway to use.

- dns-server tells clients a DNS server.

- lease 7 sets lease duration to 7 days (optional).

PC configuration (Packet Tracer GUI)

- On each PC: Desktop → IP Configuration → select DHCP. Wait a few seconds.

Verification (commands & GUI)

- On Router:

  - show ip dhcp binding — lists active leases. Expected lines like:

  - IP address        Client-ID/Lease

  - 10.10.20.11       0100.50b6.xxxxxx

  - 10.10.20.12       0100.50b6.yyyyyy

  - show ip dhcp pool — shows pool usage (addresses used / free).

- On PC:

  - Desktop → Command Prompt → ipconfig — should show IPv4 = 10.10.20.11 (or .12 etc), Mask = 255.255.255.0, Gateway = 10.10.20.1, DNS = 8.8.8.8.

  - ping 10.10.20.1 (gateway) → should reply.

  - ping 10.10.20.12 (another PC) → should reply.

Troubleshooting checklist

- If PC shows 169.254.x.x:

  - Check show ip dhcp binding — empty? DHCP not serving → verify pool exists.

  - Check switch and router interface status: show ip interface brief → Gi0/0 must be up up.

  - Did you exclude the wrong range? Make sure the pool network is correct.

- If gateway not reachable: ensure no shutdown on Gi0/0 and correct cable type.