# NAT (Static and Dynamic)

Question: Configure Static NAT and Dynamic NAT (PAT) for internal and external access.

## Topology Flowchart:

[Internal LAN 10.10.10.0/24] ─ (G0/0) [Router] (G0/1) ─ [Public Network 200.1.1.0/24]

## Steps:

Static NAT maps one inside IP to one public IP.
Dynamic NAT (PAT) allows many private IPs to share one public IP.

## Router CLI Configuration:

! Static NAT
enable
configure terminal
interface g0/0
 ip address 10.10.10.1 255.255.255.0
 ip nat inside
 no shutdown
exit
interface g0/1
 ip address 200.1.1.1 255.255.255.0
 ip nat outside
 no shutdown
exit
ip nat inside source static 10.10.10.10 200.1.1.10
end
write

! Dynamic NAT (PAT)
enable
configure terminal
access-list 1 permit 10.10.10.0 0.0.0.255
ip nat inside source list 1 interface g0/1 overload
end
write

## Verification:

Ping 200.1.1.10 from public PC for Static NAT; ping external IPs from inside for PAT.

## Concept:

Static NAT gives 1-to-1 mapping; Dynamic NAT (PAT) gives many-to-one mapping using ports.

2) NAT Lab — Static NAT (server accessible externally) + PAT for LAN hosts (Q10 & 14)

Goal

- Map internal web server (private) to a public IP (Static NAT) so external users can reach it.

- Allow internal PCs to access external internet using a single public IP (PAT / overload).

Topology (ASCII flowchart)

Inside:                          Outside:

[PC1..PC4]     [Server0]           [Public PC]

│             │                  │

└─ [Switch0] ──┴─ (Gi0/0) Router0 (Gi0/1) ─ [ISP Switch] ─ [PublicPC 200.1.1.100]

- Switch0 connects internal PCs and Server0.

- Router0 Gi0/0 → LAN (inside). Router0 Gi0/1 → Public ISP network (outside).

- Public PC (200.1.1.100) sits on outside network to test access.

IP plan

- Inside LAN: 10.10.40.0 /24

  - Router inside (gateway): 10.10.40.1

  - Web server (private static): 10.10.40.10

  - Internal PCs (manual or DHCP): e.g., 10.10.40.11–10.10.40.14

- Outside (public): 200.1.1.0 /24

  - Router outside IP: 200.1.1.2

  - Public IP assigned to server (Static): 200.1.1.10 (mapped to 10.10.40.10)

  - Public test PC: 200.1.1.100

Step-by-step (Packet Tracer actions)

- Place Router0, Switch0, Server0, 4 PCs, PublicPC.

- Connect internal devices to Switch0, Switch0 → Router0 Gi0/0. Connect Router0 Gi0/1 → PublicPC (or ISP switch).

- Configure Server0 to run HTTP (Server device → Services → HTTP ON).

Router CLI — copy/paste

enable

configure terminal

! 1) Configure inside and outside interfaces

interface GigabitEthernet0/0

description TO_LAN

ip address 10.10.40.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

description TO_ISP

ip address 200.1.1.2 255.255.255.0

no shutdown

exit

! 2) Static NAT: map internal web server to public IP

ip nat inside source static 10.10.40.10 200.1.1.10

! 3) PAT/Overload: allow internal hosts to use router's public IP

access-list 1 permit 10.10.40.0 0.0.0.255

ip nat inside source list 1 interface GigabitEthernet0/1 overload

! 4) (Optional) If you want DHCP for internal PCs, enable DHCP pool

ip dhcp excluded-address 10.10.40.1 10.10.40.10

ip dhcp pool LAN

network 10.10.40.0 255.255.255.0

default-router 10.10.40.1

end

write

Explanation — plain English

- Interface configs set gateway (inside) and public IP (outside).

- ip nat inside source static 10.10.40.10 200.1.1.10 creates a 1:1 mapping. External packets to 200.1.1.10 are forwarded to 10.10.40.10, and source/destination are translated accordingly.

- ACL 1 selects which internal addresses to translate for PAT (10.10.40.0/24).

- ip nat inside source list 1 interface GigabitEthernet0/1 overload enables PAT — many inside hosts share 200.1.1.2 (router outside IP) using different source ports.

- DHCP block optional — used if you want automatic internal addressing.

Server setup (Packet Tracer)

- Server0 → Desktop → IP Configuration:

  - IP: 10.10.40.10, Mask: 255.255.255.0, Gateway: 10.10.40.1

- Server0 → Services → HTTP → ON. Optionally edit index.html message.

Internal PC setup

- Either static set or DHCP if you enabled pool. Example PC1: 10.10.40.11 /24 GW 10.10.40.1

Verification (commands & GUI)

- Interface status

  - show ip interface brief — expect Gi0/0 10.10.40.1 up up; Gi0/1 200.1.1.2 up up.

- Static NAT verification

  - From PublicPC (200.1.1.100) → ping 200.1.1.10 → should reply (if server responds to ICMP).

  - From PublicPC in browser → http://200.1.1.10 → should load server page.

  - On Router: show ip nat translations — static entry appears (shows the static mapping).

- PAT verification

  - From internal PC (10.10.40.11) → ping 200.1.1.100 (PublicPC) → should reply if PublicPC replies.

  - On Router: show ip nat translations — dynamic entries appear similar to:

  - tcp 10.10.40.11:12345 200.1.1.100:80 -> 200.1.1.2:54321

- Show NAT config

  - show running-config | include ip nat or show ip nat statistics to view NAT counts.

Troubleshooting checklist

- If outside cannot reach internal server:

  - Verify static NAT exists: show run | include ip nat inside source static

  - Server must have correct default gateway (10.10.40.1).

  - Check firewall (none in Packet Tracer by default) or server HTTP enabled.

- If internal PCs cannot reach outside:

  - Ensure PAT command used correct outside interface name.

  - Check ACL covers the inside network (access-list 1 permit 10.10.40.0 0.0.0.255).

  - Check show ip nat translations after generating traffic.

- If show ip nat translations is empty:

  - No dynamic sessions created yet — generate traffic (e.g., ping external host from internal PC).

- If ping works but HTTP fails:

  - Server’s HTTP service might be off or port blocked.